ANNEXURE -1 – TECHNICAL DATASHEET

1.00.00     FEED LIQUID PARAMETERS

| Fluid | Sulphuric Acid Solution |
| --- | --- |
| Concentration | Normally - 10% (w/w)H2SO4 ; 90%(w/w)Water The acid concentration may vary from 8% to 12% depending on plant fluctuations. |
| Inlet Flow, m3/hr | 1)4.7, Continuous, Present with 500 mg/Nm3 case. 2) 6.0, Continuous, Future with 200 mg/Nm3 case. |
|  | Adequate margin shall be considered for selection and sizing of equipment. |
|  | Supplier to keep space in layout for future case of 200 mg/Nm3 |
| Temperature, deg C | 50 – 60 deg C. |

2.00.00    ACID CONCENTRATION PLANT OUTPUT PARAMETERS

| Concentration of Product Acid | 95% (w/w)H2SO4 ; 5%(w/w)Water |
| --- | --- |
| Product Acid Flow,m3/hr | 1)0.3 m3/hr desired, (Present with 500 mg/Nm3 case). Bidder to indicate the guarantee flow. 2)0.4 m3/hr desired, (Future with 200 mg/Nm3 case). Bidder to indicate the guarantee flow. |
| Recovered water Flow, m3/hr | Water with Max 0.2%(w/w) traces of acid. |
| Product Recovered water Flow, m3/hr | 1)4.4 m3/hr desired, (Present with 500 mg/Nm3 case). Bidder to indicate the guarantee flow. 2) 5.6 m3/hr desired, (Future with 200 mg/Nm3 case). Bidder to indicate the guarantee flow. |

3.00.00 PUMPS

| Quantity | Separate specification and datasheet included for Acid Transfer Pumps, Acid Loading Pump and Recovered water pumps. All other Pumps as per process requirement. Bidder to indicate. Two (2) for Each; one working and one standby |
| --- | --- |
|  | All pumps & motors to be sized considering final capacity in future with 200 mg/Nm3 case as indicated above. |
| Fluid to be handled | Bidder to indicate as per process. |
| Service | Bidder to indicate as per process |
| Duty | Bidder to indicate as per process |
| Rated Capacity, in m3/hr | Bidder to indicate as per process. |
| Head at rated capacity | Bidder to indicate as per process. |
|  | Pump capacity shall have 15% margin over design flow and 20% margin over calculated frictional head. |
| Material of construction |  |
| Casing | Bidder to indicate |
| Impeller | Bidder to indicate |
| Shaft | Bidder to indicate |

3.00.00     COLUMNS

| Capacity | Bidder to indicate |
| --- | --- |
| Quantity | Bidder to indicate |
| Material of Construction | Bidder to indicate. |

4.00.00 HEAT EXCHANGERS / CONDENSERS

| Capacity | Bidder to indicate |
| --- | --- |
| Quantity | Bidder to indicate |
| Material of Construction | Bidder to indicate. |

5.00.00   REBOILER - CONDENSERS

| Capacity | Bidder to indicate |
| --- | --- |
| Quantity | Bidder to indicate |
| Material of Construction | Bidder to indicate. |

6.00.00 VACUUM SYSTEM

| Vacuum Pump | Two (2); one working and one standby |
| --- | --- |
| Rated Capacity | Bidder to indicate |
| Head at rated capacity | Bidder to indicate |
| Material of construction | Bidder to indicate |

7.00.00 CHILLING PLANT

| Refrigerant Compressor | Two (2) in - one working + 1 standby |
| --- | --- |
| Chilled water Pumps | Two (2) in - one working + 1 standby |
| Condenser | Water cooled as per TEMA |
| Evaporator Cooling Coil | Bidder to indicate |
| Cooling Tower | Induced Draft Type |
| Material of construction | Bidder to indicate |

8.00.00 CHEMICAL DOSING TANK

| Numbers To be provided | As required |
| --- | --- |
| Type | Vertical cylindrical, flat bottom |
| Effective capacity, in m3 | Minimum of one day’s requirement |
| Size | Bidder to indicate. |
| Material of Construction | Bidder to indicate |

9.00.00 CHEMICAL DOSING PUMP

| Number | Two (2) one working and one standby |
| --- | --- |
| Duty | Continuous. |
| Type of Pump | Positive displacement diaphragm type. |
| Design standard | API 675. |
| Rated Capacity, in LPH | Bidder to indicate |
| Head at rated capacity | Bidder to indicate. |
| Material of construction | Bidder to indicate. |
| Pressure Relief Valve | Required. |

10.00.00       UTILITIES REQUIRED

Bidder to indicate utilities required :

| Power, 415 V, 3 PH, 50 Hz | Bidder to indicate Installed Power and Operational Power requirement |
| --- | --- |
| Make up Water for Cooling Tower System, Quality & Quantity | Bidder to indicate |
| Make up Water for Chilled Water System, Quality & Quantity | Bidder to indicate |
| Instrument Air (at pressure 6.0 bar g) Quantity, Nm3/hr | Bidder to indicate |
| Steam (Kg/hr, pressure, temperature) | High pressure steam may be 5% wet steam. Supplier to estimate steam demand considering wet steam. Low pressure steam will be supplied in dry saturated condition |
|  | Bidder to indicate different steam quantities and parameters, as required. |

11.00.00       PIPING & VALVES

A standard followed for Material of Construction under various services is indicated  below :

| Sl.No. | Service | Material |
| --- | --- | --- |
| 1. | 10% H2SO4 Solution | FRP / PP VALVES / Bidder to indicate |
| 2. | 95% H2SO4 Solution | FRP / PP VALVES / Bidder to indicate |
| 3. | Recovered water | FRP / PP VALVES / Bidder to indicate |
| 4. | Steam | ASTM A 106 Gr B, Valves - CS |
| 5. | Condensate | ASTM A 106 Gr B, Valves - CS |
| 6. | Cooling Tower Make up water | ERW Carbon Steel Pipe ; IS:1239 / IS:3589. Valves – CI / CS |
| 7. | Circulating water from Cooling Tower | ERW Carbon Steel Pipe ; IS:1239 / IS:3589. Valves – CI / CS |
| 8. | Chilled water | ASTM A 106 Gr B/ Valves - CS /Bidder to indicate |
| 9. | Instrument air | Galvanised Steel Pipe to ASTM 53 Gr. B / IS:1239. Valves – SS |
| 10. | Chemical dosing | SS / PVC /PP or equivalent tubes |