SECTION – 2 – SPECIFCATION OF TANKS

CONTENTS

CLAUSE. NO.	                              DESCRIPTION	  PAGE NO.

A.		GENERAL	  1

A-a		CODES AND STANDARDS	  1

A-b		SYSTEM DESCRIPTION	  1

A-c		SCOPE OF SUPPLY	  1

A-d		SCOPE OF SERVICES	  1

A-e		EXCLUSIONS	  2

A-f		TERMINAL POINT	  2

B.		DESIGN & CONSTRUCTION REQUIREMENT	  2

C.		QUALITY CONTROL & QUALITY ASSURANCE	  5

D.		INSPECTION & TESTING	  5

ANNEXURES

ANNEXURE-2-2	:	TECHNICAL DATA SHEETS

- General

This specification covers the design, engineering, manufacture, assembly, testing at manufacturer's works, supply and delivery to project site properly packed for transportation, including shop painting, freight, transit insurance, all taxes, duties, octroi, other charges/levies as applicable, supervision of erection, testing and commissioning at site of all materials and equipment inclusive of electrical as specified and as required for various Tanks with FRP lining inside alongwith supports,manholes, ladders, foundation bolts etc.

- Codes and Standards : The design, manufacture, inspection and testing of the equipment covered under this specification shall conform, in general, to the standards and codes (latest editions) mentioned below:

- Bureau of Indian Standards (BIS). IS 803, IS 875,IS 1893.

- British Standards.BS 4994

- American National Standards (ANSI).

- Steel Pipe Flanges & Flanged Fittings (ASME B 16.5)

- ASTM. Standards for materials

- API : American Petroleum Institute API 620 /API 650

- ASME: American Society of Mechanical Engineers, for welding standards etc ASME SEC-VIII.

- In case of any contradiction between the above standards and data specification sheets, the stipulations in the data sheets shall prevail and shall be binding on the Supplier/ Bidder.

- System Description : There are tanks provided in the system for the purpose of storage of process liquid. Waste Water tank shall store waste acidic water from FGTR system. Concentrated acid tank shall store concentrated acid. Recovered water tank shall store recovered water from the acid plant.

- Scope of supply , and supervision of erection and commissioning :

The scope of supply under this specification shall be as follows :

- One (1) no. Waste Water tank.

- One (1) no. Concentrated Acid tank.

- One (1) no. Recovered Water tank.

- Scope of supply – general :

The scope supply common for all items indicated above shall be as follows :

- All these tanks shall be complete with fittings, mountings, nozzles, manholes, ladders, handrails, stairs, foundation bolts etc

- All inlet and outlet counterflanges with nuts, bolts and gaskets.

- Set of special tools and tackles.

- First fill of lubricants and oil.

- Mandatory spares if specified. Price of same shall be evaluated.

- Erection and commissioning spares.

- List of recommended spares with Unit Rate for three (3) years of trouble-free operation. The Price of Recommended spares shall not be evaluated.

- Scope of services :

The following services shall be provided by the bidder for all equipment and accessories listed above:

1) Engineering of equipment & submission of all necessary documentation, drawings, civil assignment detail and operation and maintenance manuals.

2) Inspection and testing of all equipment at manufacturer's shop / site.

3) Supply and application of painting at site/shop.

4) Packing for road transportation ,if applicable.

5) Transportation of all equipment including transit insurance up to site.

6) Supervision of Erection and commissioning of items supplied.

7) Witnessing of Performance test of equipment at site and fulfilment

Of Guaranteed Data /Parameters.

- Exclusions :

Following items are out of scope of Bidder:

- Civil foundation.

- Terminal Point :

- As per Tender Scheme Drg .

- Design and construction requirements

1)For selection and sizing of equipment the technical data sheets may be referred.

2) Material of construction – The material of construction shall be minimum as indicated in the datasheets. However the bidder may select higher grade based on requirement of the specific function as deemed suitable.

3) This specification describes Carbon Steel tanks with corrosion resistant and durable FRP( Fibre reinforced plastic ) lining inside. All materials including FRP materials and sealant in contact with contained water shall be non-toxic.

4) The scope shall include Complete assembly of Carbon Steel shell and roof, Internal FRP linings, reinforcing members, internal & external ladders and fitting nozzles for piping and foundation bolts.

5) Adequate margin shall be considered for selection and sizing of equipment.

6) Performance Guarantee –

As per technical datasheet included in this specification.

B-1)	Fabrication

- Tanks shall be suitably constructed for safe, proper and continuous storage

# of liquid and operation of agitators as described in datasheet. The design code, material standard, minimum plate thickness, size and other details shall be as per the datasheet and drawings. The plate thicknesses indicated in the data sheets/drawings are the minimum.

- The plates shall be cold rolled through plate bending machine by several

# number of passes to true curvature.

- For fabrication at the plant area, suitable place duly approved and demarcated by client shall only be used

- Tank seams shall be so positioned that they do not pass through tank

connections. For cylindrical vessels consisting of more than two sections,

# longitudinal seams shall be offset.

- Where possible, the inside seam weld shall be ground smooth, suitable for

# application of corrosion resistant primer.

- Reinforcement pads in tank connections shall be provided as per applicable

code & the reinforced connection shall be completely pre-assembled into a

shell plate.

- All welding shall be done as per latest revision of codes and standards. All

welds are to be continuous welds. Bidder shall clearly state in his offer the

# make and type of welding rods proposed by him for fabrication.

- Inside lining of tanks shall be done to ensure quality so that there is no air gap between the lining and carbon steel plate. The procedure shall be submitted to client for approval before start of manufacturing.

B-2)	Connections

- Bidder shall furnish all piping material required for the tank connections as

indicated in tender drawings.

- Unless otherwise specified, for all flanged connections Bidder shall furnish

suitable counter flanges and the necessary nuts, bolts, gasket. Blank flanges shall be provided on spare connection.

- Flange faces of all nozzles shall be machined and square with the vessel

centerlines.

- Unless otherwise stated bolts and nuts shall be of hexagonal head.

- The material and thickness of gaskets shall be suitable for the specified

service. On completion of hydraulic test, Bidder shall replace the gaskets

used during testing at his own cost.

- The vessel and tanks shall be provided with accessories as indicated in the

GA drawing and datasheet. All the accessories shall conform to relevant

standard.

B-3)	Installation

- For installation of tank, the recommendation, regarding installation in relevant standards shall be followed in general. In addition to those recommendation, the following shall also be taken care of :

- All fabricated part, before assembly, shall be transported by the Bidder by

his own arrangement to tank installation site. All preliminary work and fabrication in part or full shall be done at the Bidder’s fabrication yard or shop as much as possible to suit his transportation arrangement subject to Purchaser’s approval.

- All material before final installation over the foundation at the respective

locality shall be inspected and faired as necessary to ensure that any damage received during transportation is corrected before erection to the satisfaction of the Engineer. Particular attention shall be given towards removal of buckles and other form of distortion in shell and bottom plates. Irregularities and dirt which would prevent metal to metal contact at the jointing faces shall be removed.

B-3)	Welding

- Welding shall be in accordance with the requirement of

ASME SEC-VIII, API 620 /API 650 as may be applicable.

- Welding sequence shall be adopted in such a way so as to minimise the

distortion due to welding shrinkage. Bidder shall indicate in his drawing the sequence of welding proposed by him which should meet prior approval of the Engineer.

- Electrodes to be used for welding shall be as per the applicable IS Code and

approved by the Engineer prior to use. Manufacturer’s test certificate for guaranteed performance shall have to be provided when called for.

- Inspection of welds shall be carried out in accordance with IS-823 or         equivalent standards

B-3)	Special Cleaning, protection, painting.

- After fabrication tank surfaces shall be cleaned thoroughly in accordance with   SSPC-SP5, White Metal Blast Cleaning to remove all loose dirt, rust, mill scales and any deleterious material.

- All exterior surfaces shall be cleaned of grease, dirt, weld spatter etc.

- Interior surfaces shall be lined as specified in the datasheet and exterior   surfaces shall be coated with the primer and finish paints in required number of coats as   specified in the datasheet/ drawings. The procedure preparation of the surfaces before application of any protective coating/paint shall be subject to   the approval of the Purchaser. Procedure for internal lining shall be subject to approval of the Purchaser.

- Quality Control Plan (QCP)/ Quality Assurance Plan (QAP)

The Supplier shall be responsible for assuring that his and his sub-suppliers quality control program meets the specified requirements.

The Supplier shall develop a recommended Purchaser inspection point program. This program shall include the design, manufacturing, inspection and testing operations that the Supplier believes may be of interest to Purchaser in demonstrating product quality, whether performed in his own or his sub-suppliers' facilities.  The inspection point program shall be submitted to the Purchaser/Purchaser's Engineer within four weeks of Letter of Intent being issued.

Purchaser and/or his designated representative shall have full access to the Supplier's and sub Supplier's facilities and work area for reviewing conformance to the approved QCP/QAP and records and for witnessing of inspections and tests. Purchaser/Purchaser's Engineer shall be notified ten (10) working days if inspection is in India and twenty one (21) working days if inspection to be carried outside India prior to the start of the tests and inspection points as specified by Purchaser.

Supplier shall have methods in place to assure that items and services including subcontracted items and services, comply with this Specification.  Supplier shall describe these methods in his proposal.

All manufacturing, processing, testing and inspection operations affecting the equipment or material may be subject to surveillance by Purchaser or Purchaser's Engineer.

Prior to production, Supplier shall submit to Purchaser a fabrication sequence describing inspection and/or tests to be performed, for use in determining inspection points that Purchaser may desire to witness.  The Purchaser shall advise the Supplier, prior to production, of those fabrication steps and shop inspection points that the Purchaser desires to witness, and shall identify which of these points require work to be stopped pending written authorization to proceed from Purchaser/Purchaser's Engineer.

For fabrication steps and tests that Purchaser desires to witness, Supplier shall give ten (10) working days if in India and twenty one (21) working days if in abroad for notice of witness points, and fifteen (15) working days if in India and twenty eight (28) working days if in abroad for notice of hold points.  Notification of hold or witness points shall not be considered to have been given until Purchaser has acknowledged receipt of such notification.

In the event of non-conformance of materials or equipment, the Supplier is to prepare a `Non-conformance Report'.  This will be signed jointly by the Supplier and the Purchaser's Engineer.  Each Non-conformance Report shall be dealt with on a `VERY URGENT' basis.  All deviations from this Specification must be documented and referred to Purchaser for written disposition.

The Supplier shall indicate the name of the manufacturer and place of test and inspection for each equipment supplied by him in the offer.

- Inspection and Testing

# Given below a Standard Inspection and Testing requirement (ITR). The relevant test for the specified equipment shall be applicable as per this document

1)	The Supplier shall conduct all tests and inspections required to ensure that the equipment furnished shall conform to the requirements of this specification and in compliance with requirements of applicable Codes and Standards. The particulars of the proposed tests and the procedures for the tests shall be submitted to the Purchaser for approval before conducting the tests. The Purchaser's representative shall be given full access to all tests. The Supplier shall intimate the Purchaser well in advance (at least 15 days notice) so that if desired his representative can witness the test.

2)	Where stage inspection is to be witnessed by Purchaser, in addition to above, the Supplier shall submit to the Purchaser at the initiation of the contract, the detailed PERT-Chart showing the manufacturing programme and indicating the period where Purchaser or his authorised inspecting agency is required at the shop.

3)	Shop Tests

3-1)Material Test

i) 	All materials used for construction shall be of tested quality. Physical and Chemical tests of materials shall be conducted as per the relevant standards and test certificates shall be made available to the Purchaser. If demanded by Purchaser’s inspector, proof of use of only tested quality material shall be furnished.

ii) 	Where stage inspection is to be witnessed by Purchaser, all material test certificates shall be correlated and verified with the actual material used for construction by Purchaser’s inspector who shall stamp the material. In case mill test certificates for the materials are not available, the Supplier shall carry out physical and chemical tests at his own cost from a testing agency, approved by the Purchaser, as per the requirement of specified material standard. The samples for physical and chemical testing shall be drawn up in presence of Purchaser’s inspector who shall also witness the testing.

iii) 	All castings shall be sound, clean and free from porosity, blow holes, hard spots and other harmful defects.

iv) 	The equipment system components shall be subjected to non-destructive testing (NDT) like Radiography, Ultrasonic test, Magnetic particle test, Liquid penetration test etc. as required & QAP must be approved by the Purchaser before  all tests. The accepted limit shall be as stipulated therein. All components subjected to NDT shall be identified and stamped by Purchaser’s inspector on successful completion of testing.

v)	Control panel sequence tests to be done.

- During assembly all clearances and alignments shall also be checked and recorded.

- All major components shall be tested for chemical composition and mechanical strength.

3-2) Hydraulic Test

All pressure parts shall be hydraulically tested at shop as per relevant code and at not less than 150% of design pressure where not specified, prior to painting and lining, if applicable. The pressure parts shall be kept pressurised for at least one (1) hour at this test pressure, unless contradicted by the relevant  test code.

3-3)Dynamic Balancing Test

All rotating components and the assembly of the shall be subjected to dynamic balancing test, as applicable.

3-4)Test for capacity, pressure drop and efficiency of equipment shall be made as per manufacturer’s standard.

3-5)Performance Test at shop

Type test/routine test for the system / equipment shall be done for satisfactory performance in accordance with the relevant IS/BS/ASME or equivalent standard.

Tests shall be conducted with actual driver being furnished.

Reports and test certificates of the above tests shall be submitted to the Purchaser.

3-6) All drive motors shall be tested as per Electrical standards and codes.

4)Performance Test at Site – Given below are the general requirement of site test.

Applicable items shall be considered.

4-1) After the equipment along with all accessories are installed at site, the following tests shall be conducted at site by the supplier with his own calibrated testing equipment:

- To check capability of each equipment to deliver rated performance specified & guaranteed by the supplier.

- To check proper and continuously reliable operation of the control/regulation/sequence of the equipment furnished.

c)	To check proper operation, freedom from excessive temperature rise, unsatisfactory vibration noise.

d)	The tests shall be carried out for a minimum period of eight (8) hours or as directed by the Engineer.

e)	To check the loaded and unloaded shaft horse power (BHP)

f)	To check power (KW) drawn by the equipment

g)	The piping system shall be subjected to a leak test at 1.5 times the design pressure.

If the performance at field is found not to meet the requirement, the liquidated damages shall be applied.